Unit- 4 Renewable Energy

Energy Basics — Conventional Energy

1. Energy

Definition: Energy is the capacity to do work.

Energy is required for:

- transportation

- electricity generation

- industrial processes

- domestic activities

Example: electricity, heat, fuel energy.

2. Conventional Energy

Definition : Conventional energy refers to traditional energy sources that have been used for a long time to meet human energy needs.

These sources are mainly fossil fuels and are usually non-renewable.

They were the primary sources of energy during industrial development.

Main Characteristics of Conventional Energy

| Characteristic | Explanation |
| --- | --- |
| Traditional sources | Used for many decades |
| Mostly non-renewable | Limited availability |
| High energy output | Large-scale electricity generation |
| Environmental impact | Causes pollution and greenhouse gases |

Examples of Conventional Energy

1. Coal : Coal is a fossil fuel formed from plant remains buried underground for millions of years.

Uses

- thermal power plants

- steel production

- cement industry

Advantages

- abundant in many countries

- high energy content

Disadvantages

- air pollution

- carbon dioxide emissions

2. Petroleum (Crude Oil)

Petroleum is a liquid fossil fuel found underground and refined into many products.

Common Products

- petrol

- diesel

- kerosene

- lubricants

Uses

- transportation fuel

- petrochemical industries

- electricity generation

Problems

- air pollution

- limited reserves

3 Natural Gas

Natural gas is a gaseous fossil fuel mainly composed of methane.

Uses

- cooking fuel (LPG / PNG)

- electricity generation

- fertilizer production

Advantages

- cleaner than coal and petroleum

- high efficiency

Disadvantages

- still produces greenhouse gases

- limited supply

Renewable Energy

1 Definition : Renewable energy is energy obtained from natural sources that are continuously replenished and cannot be exhausted by human use.

These energy sources are naturally renewed through environmental processes.

Examples include:

- solar energy

- wind energy

- hydro energy

- biomass energy

- geothermal energy

2. Meaning of Renewable Resources

Renewable resources are natural resources that can be replaced or regenerated naturally within a short period of time.

They are sustainable because their supply is continuously renewed by nature.

Examples of Renewable Resources

| Resource | Source |
| --- | --- |
| Solar energy | Sunlight |
| Wind energy | Moving air |
| Hydropower | Flowing water |
| Biomass | Organic matter |
| Geothermal energy | Heat from Earth |

Conventional Energy vs Renewable Energy

| Feature | Conventional Energy | Renewable Energy |
| --- | --- | --- |
| Definition | Traditional energy sources used for a long time | Energy obtained from naturally replenished resources |
| Availability | Limited and exhaustible | Abundant and continuously replenished |
| Examples | Coal, petroleum, natural gas | Solar, wind, hydro, biomass |
| Pollution | Produces high pollution and greenhouse gases | Very low or no pollution |
| Sustainability | Not sustainable for long-term use | Sustainable and environmentally friendly |
| Cost | High environmental cost | Lower environmental impact |

Need for Renewable Energy

Renewable energy is important to meet the growing energy demand while protecting the environment and natural resources.

1. Energy Crisis

Meaning : An energy crisis occurs when the demand for energy exceeds the available supply of energy resources.

Most conventional energy sources such as coal, petroleum, and natural gas are limited and non-renewable. Rapid industrialization and population growth increase energy consumption, leading to shortages.

Role of Renewable Energy

Renewable energy sources such as solar, wind, and hydro energy provide alternative energy and help reduce dependence on fossil fuels.

2. Pollution Reduction

Conventional energy sources produce large amounts of air pollution and greenhouse gases.

Examples:

- carbon dioxide (CO₂)

- sulfur dioxide (SO₂)

- nitrogen oxides (NOₓ)

These pollutants cause:

- global warming

- climate change

- respiratory diseases

Benefit of Renewable Energy

Renewable energy sources produce little or no pollution, helping to protect the environment and improve air quality.

3. Sustainability

Meaning : Sustainability refers to the ability to meet present energy needs without compromising the ability of future generations to meet their own needs.

Fossil fuels are exhaustible, meaning they will eventually run out.

Renewable energy sources are naturally replenished, making them suitable for long-term energy supply.

Summary Table

| Reason | Explanation |
| --- | --- |
| Energy crisis | Renewable energy helps overcome shortage of fossil fuels |
| Pollution reduction | Produces less pollution compared to conventional energy |
| Sustainability | Provides long-term and continuous energy supply |

Solar Energy

Definition : Solar energy is the energy obtained from the sun’s radiation and converted into useful forms such as electricity or heat.

Solar energy is a renewable and clean source of energy because sunlight is continuously available and does not produce pollution.

Characteristics of Solar Energy

| Feature | Explanation |
| --- | --- |
| Renewable | Sunlight is continuously available |
| Clean energy | Produces no air pollution |
| Abundant | Available in most parts of the world |
| Sustainable | Can be used for long-term energy supply |

Solar Cell

Definition: A solar cell is a device that converts sunlight directly into electrical energy using the photovoltaic effect.

Solar cells are the basic units used in solar panels.

Principle of Solar Energy (Photovoltaic Effect)

Solar cells work on the photovoltaic effect, in which light energy is converted into electrical energy.

When sunlight falls on a semiconductor material (usually silicon), electrons gain energy and start moving, producing electric current.

Working of a Solar Cell

The working process occurs in several steps:

Step 1 — Sunlight Absorption

Sunlight falls on the solar panel containing photovoltaic cells.

Step 2 — Electron Excitation

Photons from sunlight transfer energy to electrons in the semiconductor material.

Electrons become energized and move freely.

Step 3 — Electric Field Action

The p–n junction inside the solar cell creates an electric field.

This field forces electrons to move in a particular direction.

Step 4 — Electric Current Generation

The movement of electrons through an external circuit produces direct current (DC) electricity.

Step 5 — Power Utilization

The electricity generated can be used for:

- lighting

- charging batteries

- running electrical appliances

Components of a Solar Cell

| Component | Function |
| --- | --- |
| Glass cover | Protects the cell |
| Anti-reflective coating | Increases sunlight absorption |
| Semiconductor layer | Generates electric charge |
| Metal contacts | Collect and transfer electricity |

Applications of Solar Energy

Solar energy is used in many areas:

- solar panels for electricity

- solar water heaters

- solar street lights

- solar cooking systems

- satellites and space stations

Advantages of Solar Energy

| Advantage | Explanation |
| --- | --- |
| Renewable | Unlimited energy source |
| Environment friendly | No pollution |
| Low operating cost | Sunlight is free |
| Widely available | Accessible in many regions |

Limitations of Solar Energy

| Limitation | Explanation |
| --- | --- |
| Weather dependent | Requires sunlight |
| High installation cost | Solar panels are expensive |
| Energy storage required | Batteries needed for night use |

Solar Energy — Advantages, Limitations, Applications

Advantages

| Advantage | Explanation |
| --- | --- |
| Clean energy | Solar energy does not produce harmful gases or air pollution. |
| Abundant | Sunlight is widely available and renewable. |
| Low operating cost | After installation, maintenance and running costs are very low. |

Limitations

| Limitation | Explanation |
| --- | --- |
| High installation cost | Initial cost of solar panels and equipment is high. |
| Weather dependency | Energy production depends on sunlight and weather conditions. |

Applications

| Application | Description |
| --- | --- |
| Solar panels | Used to generate electricity from sunlight. |
| Solar water heaters | Used for heating water using solar energy. |
| Solar farms | Large-scale solar power plants that produce electricity for cities and industries. |

Top of Form

Bottom of Form

Wind Energy

Definition : Wind energy is the energy obtained from the movement of air (wind) and converted into useful forms such as electricity using wind turbines.

Wind energy is a renewable and clean source of energy because wind is naturally produced by atmospheric processes and does not get exhausted.

Principle of Wind Energy

Wind energy works on the principle of conversion of kinetic energy of moving air into mechanical energy and then into electrical energy.

Steps involved in the principle:

- Wind flows due to difference in air pressure caused by uneven heating of the Earth by the sun.

- Moving air contains kinetic energy.

- When wind strikes the blades of a turbine, it causes them to rotate.

- The rotating blades turn a shaft connected to a generator.

- The generator converts mechanical energy into electrical energy.

Wind Turbines : A wind turbine is a machine that converts the kinetic energy of wind into mechanical energy and then into electrical energy.

Wind turbines are usually installed in wind farms where wind speed is high and consistent.

Main Components of a Wind Turbine

| Component | Function |
| --- | --- |
| Blades | Capture the kinetic energy of wind |
| Rotor | Rotates when blades move |
| Shaft | Transfers mechanical energy |
| Generator | Converts mechanical energy into electricity |
| Tower | Supports turbine at high altitude |
| Nacelle | Contains gearbox and generator |

Working of Wind Turbines

The working process occurs in several stages:

Step 1 — Wind Flow

Wind moves across the turbine blades.

Step 2 — Blade Rotation

The wind force causes the blades to rotate, turning the rotor.

Step 3 — Shaft Rotation

The rotor is connected to a shaft, which rotates due to blade movement.

Step 4 — Generator Operation

The rotating shaft spins a generator, converting mechanical energy into electricity.

Step 5 — Electricity Transmission

Generated electricity is transferred to power grids for use in homes and industries.

Applications of Wind Energy

Wind energy is widely used for:

- electricity generation

- wind farms

- water pumping

- agricultural irrigation

- remote power supply systems

Advantages of Wind Energy

| Advantage | Explanation |
| --- | --- |
| Renewable | Wind is naturally replenished |
| Clean energy | Produces no air pollution |
| Low operating cost | No fuel cost |
| Sustainable | Can be used long-term |

Limitations of Wind Energy

| Limitation | Explanation |
| --- | --- |
| Weather dependent | Requires sufficient wind speed |
| Noise pollution | Turbine operation produces sound |
| Land requirement | Wind farms need large areas |

Hydropower (Hydroelectric Energy)

Definition : Hydropower is the energy produced by using the force of flowing or falling water to generate electricity.

It is a renewable source of energy because the water cycle continuously replenishes water resources.

Hydropower is widely used in hydroelectric power plants.

Principle of Hydropower Plant

Hydropower plants work on the principle of conversion of potential energy of stored water into mechanical energy and then into electrical energy.

Steps of the principle:

- Water is stored in a dam at a higher elevation.

- Stored water has potential energy.

- When water flows down through pipes (penstocks), its potential energy converts into kinetic energy.

- Flowing water strikes the turbine blades, causing them to rotate.

- The rotating turbine drives a generator.

- The generator converts mechanical energy into electrical energy.

Hydropower Plant

A hydropower plant is a facility that generates electricity using the energy of flowing water.

Main Components of a Hydropower Plant

| Component | Function |
| --- | --- |
| Dam | Stores water at high level |
| Reservoir | Large water storage behind dam |
| Penstock | Pipe carrying water to turbine |
| Turbine | Rotated by flowing water |
| Generator | Converts mechanical energy into electricity |
| Transmission lines | Carry electricity to consumers |

Working of a Hydropower Plant

Step 1 — Water Storage

Water is stored in a reservoir behind the dam.

Step 2 — Water Flow

Water is released from the reservoir through penstocks.

Step 3 — Turbine Rotation

Flowing water strikes the turbine blades, causing them to rotate.

Step 4 — Generator Operation

The turbine rotates a generator, producing electricity.

Step 5 — Power Distribution

Electricity is transmitted through power lines to homes and industries.

Energy Conversion in Hydropower

| Energy Type | Conversion |
| --- | --- |
| Potential energy | Stored water in dam |
| Kinetic energy | Flowing water |
| Mechanical energy | Rotating turbine |
| Electrical energy | Generator output |

Applications of Hydropower

Hydropower is used for:

- large-scale electricity generation

- irrigation projects

- flood control

- water supply systems

Advantages of Hydropower

| Advantage | Explanation |
| --- | --- |
| Renewable | Water cycle continuously replenishes supply |
| Clean energy | Produces very low pollution |
| Reliable | Stable electricity generation |
| Low operating cost | No fuel required |

Limitations of Hydropower

| Limitation | Explanation |
| --- | --- |
| High construction cost | Dam construction is expensive |
| Environmental impact | Affects aquatic ecosystems |

Hydropower — Advantages, Limitations, Examples

Advantages

| Advantage | Explanation |
| --- | --- |
| Reliable | Hydropower plants can generate electricity continuously when sufficient water is available. |
| Renewable | Water is replenished through the natural water cycle, making hydropower a renewable energy source. |

Limitations

| Limitation | Explanation |
| --- | --- |
| Ecosystem disruption | Construction of dams can disturb aquatic life and natural river ecosystems. |
| High construction cost | Building dams and hydropower plants requires large initial investment and infrastructure. |

Examples

| Example | Description |
| --- | --- |
| Hydroelectric dams | Large dams built across rivers to generate electricity using flowing water. |

Biomass Energy

Definition : Biomass energy is the energy obtained from organic materials such as plants, agricultural waste, animal waste, and other biological matter.

These materials are collectively called biomass, and they can be used as fuel to produce heat, electricity, or biofuels.

Biomass energy is considered renewable because the organic materials used can be regenerated through natural processes.

Meaning of Biomass

Biomass refers to organic matter derived from living or recently living organisms.

Examples of biomass materials:

- wood and plant residues

- agricultural waste (crop residues)

- animal manure

- food waste

- forest residues

These materials contain stored chemical energy from the sun, which plants absorb during photosynthesis.

Principle of Biomass Energy

Biomass energy works on the principle of conversion of chemical energy stored in organic matter into heat or electricity.

Plants absorb solar energy through photosynthesis, storing it in the form of chemical energy. When biomass is burned or processed, this energy is released and used for energy production.

Conversion Methods of Biomass Energy

Biomass energy can be converted into usable energy through different methods.

| Method | Explanation |
| --- | --- |
| Direct combustion | Biomass is burned to produce heat and electricity |
| Anaerobic digestion | Organic waste is decomposed by microorganisms to produce biogas |
| Fermentation | Biomass is converted into biofuels such as ethanol |
| Gasification | Biomass is converted into combustible gas |

Working of Biomass Energy Production

Step 1 — Collection of Biomass

Organic materials such as crop waste, wood, or animal manure are collected.

Step 2 — Processing of Biomass

The biomass is processed through combustion, fermentation, or digestion.

Step 3 — Energy Conversion

The stored chemical energy in biomass is converted into heat or gas fuel.

Step 4 — Electricity Generation

Heat or biogas is used to run turbines or generators, producing electricity.

Biomass Energy Systems

Examples include:

- biomass power plants

- biogas plants

- biofuel production systems

Advantages of Biomass Energy

| Advantage | Explanation |
| --- | --- |
| Renewable | Biomass resources can be regenerated |
| Waste management | Uses agricultural and organic waste |
| Reduces landfill waste | Converts waste into useful energy |
| Lower carbon emissions | Emits less carbon compared to fossil fuels |

Limitations of Biomass Energy

| Limitation | Explanation |
| --- | --- |
| Air pollution | Burning biomass releases smoke and gases |
| Land use | Large biomass production requires land |
| Lower efficiency | Less efficient compared to fossil fuels |

Applications of Biomass Energy

Biomass energy is used for:

- electricity generation

- biogas production for cooking

- biofuel production (ethanol, biodiesel)

- heating systems

Biomass Energy — Sources, Advantages, Limitations, Applications

Sources of Biomass Energy

| Source | Explanation |
| --- | --- |
| Agricultural waste | Crop residues such as rice husk, straw, and corn stalks used as fuel. |
| Animal waste | Animal manure used to produce biogas through anaerobic digestion. |
| Wood | Wood and forest residues used for heating and electricity generation. |

Advantages

| Advantage | Explanation |
| --- | --- |
| Renewable | Biomass materials can be replenished through natural processes. |
| Waste utilization | Converts agricultural and organic waste into useful energy. |

Limitations

| Limitation | Explanation |
| --- | --- |
| Air pollution if poorly managed | Burning biomass inefficiently can release smoke and harmful gases. |

Applications

| Application | Description |
| --- | --- |
| Biogas plants | Convert animal waste and organic materials into biogas for cooking and electricity. |
| Biomass power plants | Generate electricity by burning biomass fuels such as wood and agricultural residues. |

Geothermal Energy

Definition : Geothermal energy is the energy obtained from the heat stored inside the Earth.

The word geothermal comes from:

- Geo = Earth

- Thermal = Heat

This heat from the Earth's interior can be used to generate electricity or provide heating.

Geothermal energy is considered a renewable and sustainable energy source because the Earth's internal heat is continuously produced.

Principle of Geothermal Energy

Geothermal power plants work on the principle of using heat from the Earth’s interior to produce steam, which drives turbines to generate electricity.

The process involves converting thermal energy from underground reservoirs into mechanical energy and then electrical energy.

Source of Geothermal Heat

The heat inside the Earth comes from:

- radioactive decay of minerals

- heat remaining from Earth’s formation

- magma movement beneath the Earth's crust

This heat warms underground water and forms hot water reservoirs and steam.

Geothermal Power Plant

Main Components of a Geothermal Power Plant

| Component | Function |
| --- | --- |
| Geothermal reservoir | Underground source of hot water or steam |
| Production well | Brings hot water or steam to the surface |
| Turbine | Rotated by steam to produce mechanical energy |
| Generator | Converts mechanical energy into electricity |
| Injection well | Returns cooled water back underground |

Working of Geothermal Power Plant

Step 1 — Heat from Earth's Interior

Heat from underground rocks warms water stored beneath the Earth's surface.

Step 2 — Steam Production

The heated water forms high-pressure steam.

Step 3 — Steam Movement

Steam rises through production wells to the surface.

Step 4 — Turbine Rotation

Steam rotates the turbine blades.

Step 5 — Electricity Generation

The rotating turbine drives a generator, producing electricity.

Step 6 — Water Re-injection

After steam cools, water is pumped back underground through injection wells.

This maintains the geothermal reservoir.

Applications of Geothermal Energy

Geothermal energy is used for:

- electricity generation

- heating buildings

- greenhouse heating

- industrial processes

- hot water supply

Advantages of Geothermal Energy

| Advantage | Explanation |
| --- | --- |
| Renewable | Earth continuously produces heat |
| Low pollution | Produces very low greenhouse gases |
| Reliable | Works continuously unlike solar or wind |
| Small land requirement | Requires less land compared to other power plants |
| Continuous energy supply | Geothermal energy is available continuously because heat from the Earth’s interior is constantly produced. |

Limitations of Geothermal Energy

| Limitation | Explanation |
| --- | --- |
| Location specific | Only available in geothermal regions |
| High initial cost | Drilling wells is expensive |
| Possible gas emissions | Small amounts of gases may be released |

Limitations

| Limitation | Explanation |
| --- | --- |
| Location dependent | Geothermal power plants can only be built in areas where geothermal heat sources are available, such as volcanic or tectonically active regions. |

Applications

| Application | Description |
| --- | --- |
| Geothermal power plants | Facilities that use heat from underground reservoirs to produce steam and generate electricity. |

Tidal Energy

Definition : Tidal energy is the energy obtained from the rise and fall of ocean tides, which is used to generate electricity.

Tides occur due to the gravitational attraction of the Moon and the Sun on Earth's oceans.

Tidal energy is a renewable and predictable source of energy because tides occur regularly.

Principle of Tidal Energy

Tidal power plants work on the principle of converting the kinetic and potential energy of moving seawater during tides into electrical energy.

During high and low tides, large volumes of water move in and out of coastal areas. This moving water rotates turbines, which drive generators to produce electricity.

Tidal Power Plant:

Main Components of a Tidal Power Plant

| Component | Function |
| --- | --- |
| Tidal barrage | Dam built across an estuary or coastal area |
| Turbine | Rotated by moving tidal water |
| Generator | Converts mechanical energy into electricity |
| Sluice gates | Control water flow during tides |
| Powerhouse | Contains turbine and generator |

Working of Tidal Energy System

Step 1 — High Tide

During high tide, seawater enters the basin behind the tidal barrage.

Step 2 — Water Storage

Water is temporarily stored in the tidal basin.

Step 3 — Water Flow

When the tide recedes, stored water flows back toward the sea through turbines.

Step 4 — Turbine Rotation

The flowing water rotates the turbine blades.

Step 5 — Electricity Generation

The turbine drives a generator, producing electricity.

Types of Tidal Energy Systems

| Type | Explanation |
| --- | --- |
| Tidal barrage | Dam across tidal estuary |
| Tidal stream generator | Underwater turbines in fast tidal currents |
| Tidal lagoon | Artificial lagoon capturing tidal energy |

Advantages of Tidal Energy

| Advantage | Explanation |
| --- | --- |
| Renewable | Tides occur naturally and continuously |
| Predictable | Tidal cycles are highly predictable |
| Clean energy | Produces very little pollution |
| Predictable energy source | Tides occur regularly due to gravitational forces of the Moon and Sun, making tidal energy highly predictable. |

Limitations of Tidal Energy

| Limitation | Explanation |
| --- | --- |
| High construction cost | Building tidal barrages is expensive |
| Location dependent | Suitable only in coastal areas with large tidal ranges |
| Environmental impact | May affect marine ecosystems |
| High infrastructure cost | Construction of tidal barrages and underwater turbines requires large investment and complex infrastructure. |

Applications of Tidal Energy

Tidal energy is used for:

- electricity generation in coastal regions

- tidal power stations

- small-scale coastal energy systems

Green Hydrogen

Definition : Green hydrogen is hydrogen fuel produced using renewable energy sources such as solar, wind, or hydropower to split water into hydrogen and oxygen through electrolysis.

It is called green because the process does not produce carbon emissions.

Green hydrogen is considered a clean and sustainable energy carrier.

Meaning of Hydrogen as Energy

Hydrogen is the most abundant element in the universe and can be used as a fuel.

When hydrogen is used as fuel, it produces:

H2+O2→H2O+Energy

The only by-product is water, making it environmentally friendly.

Principle of Green Hydrogen Production

Green hydrogen is produced using the electrolysis of water powered by renewable energy.

Electrolysis splits water into hydrogen and oxygen using electricity.

Steps of the principle:

- Renewable energy (solar or wind) produces electricity.

- Electricity is supplied to an electrolyzer.

- Water molecules split into hydrogen and oxygen.

- Hydrogen gas is collected and stored as fuel.

Electrolysis Process

Chemical reaction:

2H2O→2H2+O2

Meaning:

Water → Hydrogen + Oxygen.

Main Components of Green Hydrogen System

| Component | Function |
| --- | --- |
| Renewable energy source | Provides electricity |
| Electrolyzer | Splits water into hydrogen and oxygen |
| Water supply | Raw material for hydrogen production |
| Hydrogen storage tanks | Store produced hydrogen |
| Fuel cells / engines | Use hydrogen to produce energy |

Working of Green Hydrogen Production

Step 1 — Renewable Electricity

Electricity is generated using solar panels or wind turbines.

Step 2 — Electrolysis

Electric current passes through water in the electrolyzer, splitting water molecules.

Step 3 — Hydrogen Production

Hydrogen gas is produced at the cathode, and oxygen is produced at the anode.

Step 4 — Storage

Hydrogen gas is collected and stored in high-pressure tanks.

Step 5 — Energy Use

Hydrogen can be used in fuel cells, vehicles, and power generation systems.

Applications of Green Hydrogen

Green hydrogen is used in:

- hydrogen fuel cell vehicles

- electricity generation

- industrial processes (steel production)

- energy storage systems

- fertilizer production

Advantages of Green Hydrogen

| Advantage | Explanation |
| --- | --- |
| Clean fuel | Produces only water as by-product |
| Renewable | Produced using renewable energy |
| Energy storage | Can store excess renewable energy |
| Versatile | Used in transport, industry, and power generation |

Limitations of Green Hydrogen

| Limitation | Explanation |
| --- | --- |
| High production cost | Electrolyzers and renewable systems are expensive |
| Energy intensive | Requires large amount of electricity |
| Storage challenges | Hydrogen storage and transport are difficult |

Importance of Green Hydrogen

Green hydrogen is important for:

- reducing carbon emissions

- replacing fossil fuels in industries

- supporting clean energy transition

Many countries are investing heavily in hydrogen energy technologies.

Why It Is Called “Green”

Hydrogen can be produced in different ways.

| Type | Production Method | Environmental Impact |
| --- | --- | --- |
| Green hydrogen | Renewable energy electrolysis | No carbon emissions |
| Grey hydrogen | Natural gas reforming | Produces CO₂ |
| Blue hydrogen | Fossil fuels with carbon capture | Reduced emissions |

Green hydrogen is the cleanest form of hydrogen energy.

Importance of Green Hydrogen

Green hydrogen helps in:

- reducing greenhouse gas emissions

- replacing fossil fuels in industries

- storing renewable energy

- supporting clean transportation

Key Advantages

| Advantage | Explanation |
| --- | --- |
| Clean fuel | Produces only water when used |
| Renewable | Produced using renewable energy |
| Energy storage | Stores excess renewable electricity |

Production of Hydrogen (Electrolysis of Water Using Renewable Electricity)

Definition : Hydrogen can be produced by electrolysis of water, a process in which electricity is used to split water molecules into hydrogen and oxygen gases.

When the electricity used for electrolysis comes from renewable energy sources such as solar, wind, or hydropower, the hydrogen produced is called green hydrogen.

Principle of Hydrogen Production

The production of hydrogen through electrolysis is based on the principle that electric current passing through water can break the chemical bonds between hydrogen and oxygen atoms.

Water contains:

H2O

When electricity flows through water, it splits into:

2H2O→2H2+O2

This means:

Water → Hydrogen gas + Oxygen gas.

Electrolysis Process

Electrolysis takes place in a device called an electrolyzer.

Main Components of Electrolysis System

| Component | Function |
| --- | --- |
| Electrolyzer | Device where water splitting occurs |
| Electrodes | Conduct electricity in water |
| Electrolyte | Conducts electric current |
| Renewable electricity source | Provides electrical energy |
| Hydrogen storage tank | Stores produced hydrogen |

Working of Electrolysis for Hydrogen Production

Step 1 — Renewable Electricity Supply

Electricity is generated from renewable sources such as solar panels or wind turbines.

Step 2 — Water Supply to Electrolyzer

Water is supplied to the electrolysis chamber.

Step 3 — Electric Current Flow

Electric current passes through the water via two electrodes (anode and cathode).

Step 4 — Water Splitting

Water molecules break into hydrogen and oxygen gases.

- Hydrogen forms at the cathode

- Oxygen forms at the anode

Step 5 — Gas Collection

The hydrogen gas is collected and stored, while oxygen may be released or used for industrial purposes.

Chemical Reactions in Electrolysis

At Cathode:

2H2O+2e-→H2+2OH-

At Anode:

4OH-→O2+2H2O+4e-

Overall reaction:

2H2O→2H2+O2

Applications of Hydrogen Produced

Hydrogen produced by electrolysis can be used for:

- hydrogen fuel cell vehicles

- electricity generation

- industrial processes

- ammonia production

- energy storage systems

Advantages of Electrolysis Using Renewable Energy

| Advantage | Explanation |
| --- | --- |
| Clean process | Produces no carbon emissions |
| Renewable | Uses solar or wind electricity |
| Energy storage | Stores excess renewable energy as hydrogen |

Limitations

| Limitation | Explanation |
| --- | --- |
| High cost | Electrolyzers and renewable systems are expensive |
| Energy intensive | Requires large amount of electricity |

Hydrogen Storage

Hydrogen storage is an important part of the hydrogen energy system because hydrogen must be stored safely before it is transported or used as fuel.

Hydrogen is a very light gas, so it requires special storage methods.

Definition : Hydrogen storage is the process of storing hydrogen gas safely and efficiently for later use as a fuel or energy source.

Hydrogen can be stored in different ways depending on the pressure, temperature, and application.

Methods of Hydrogen Storage

Two common storage methods are:

- High-pressure storage (compressed hydrogen)

- Cryogenic storage (liquid hydrogen)

1. High-Pressure Hydrogen Storage

Definition : High-pressure storage is the method of storing hydrogen gas in strong tanks at very high pressure.

Hydrogen gas is compressed and stored in cylinders or tanks.

Working Principle

Since hydrogen has very low density, compressing it increases the amount of hydrogen stored in a container.

Typical storage pressure:

- 350 bar

- 700 bar

These tanks are made of high-strength steel or carbon fiber materials.

Applications

High-pressure hydrogen storage is commonly used in:

- hydrogen fuel cell vehicles

- industrial hydrogen supply

- portable hydrogen storage systems

Advantages

| Advantage | Explanation |
| --- | --- |
| Simple technology | Easier to implement compared to other storage methods |
| Fast filling | Tanks can be filled quickly |
| Suitable for vehicles | Used in hydrogen fuel cell cars |

Limitations

| Limitation | Explanation |
| --- | --- |
| High pressure required | Needs very strong tanks |
| Safety concerns | Risk of leakage if not handled properly |

2 Cryogenic Hydrogen Storage

Definition

Cryogenic storage is the method of storing hydrogen in liquid form at extremely low temperatures.

Hydrogen gas is cooled to very low temperatures until it becomes liquid.

Temperature required:

-253∘C

Working Principle

When hydrogen is cooled to very low temperatures, it changes from gas to liquid, which greatly reduces its volume.

This allows more hydrogen to be stored in a smaller space.

Applications

Cryogenic hydrogen storage is used in:

- space rockets

- aerospace systems

- large-scale hydrogen storage systems

Advantages

| Advantage | Explanation |
| --- | --- |
| High storage density | Stores large amount of hydrogen |
| Efficient for large-scale storage | Suitable for industrial applications |

Limitations

| Limitation | Explanation |
| --- | --- |
| Extremely low temperature required | Requires special cooling systems |
| Energy intensive | Liquefaction requires large energy |
| Boil-off losses | Hydrogen may slowly evaporate |

Comparison of Hydrogen Storage Methods

| Feature | High-Pressure Storage | Cryogenic Storage |
| --- | --- | --- |
| State of hydrogen | Gas | Liquid |
| Storage condition | High pressure | Very low temperature |
| Typical use | Vehicles | Aerospace and large storage |
| Complexity | Moderate | High |

Importance of Hydrogen Storage

Hydrogen storage is essential for:

- hydrogen fuel cell vehicles

- renewable energy storage

- industrial hydrogen supply

- transportation of hydrogen fuel

1.  Transportation of Hydrogen

Definition : Hydrogen transportation refers to the process of moving hydrogen from production sites to storage facilities or places where it will be used.

Hydrogen can be transported in different forms depending on the distance, quantity, and method of storage.

Methods of Hydrogen Transportation

1. Pipeline Transport

Hydrogen gas can be transported through pipelines, similar to natural gas pipelines.

Features

- pipelines carry compressed hydrogen gas

- suitable for large-scale industrial transport

Advantages

- continuous supply

- efficient for long distances

Limitations

- expensive pipeline infrastructure

- hydrogen leakage risk due to small molecule size

2.  High-Pressure Cylinders

Hydrogen gas can be transported in high-pressure cylinders or tanks mounted on trucks.

Features

- hydrogen compressed at high pressure

- cylinders made of strong steel or carbon fiber

Applications

- hydrogen fuel stations

- laboratory supply

3.  Liquid Hydrogen Transport

Hydrogen can be transported in liquid form using cryogenic tanks.

Hydrogen must be cooled to:

-253∘ C

Advantages

- higher storage density

- suitable for large-scale transport

Limitations

- expensive cooling systems

- evaporation losses

4. Hydrogen Carriers (Ammonia or Liquid Organic Carriers)

Hydrogen can also be transported as chemical compounds.

Example:

- ammonia (NH₃)

- liquid organic hydrogen carriers (LOHC)

Later hydrogen can be extracted from these compounds.

5 Applications of Hydrogen

Hydrogen has many important applications in energy, transportation, and industry.

6 Fuel Cell Vehicles

Hydrogen is used in fuel cells to power vehicles.

Examples:

- hydrogen cars

- buses

- trucks

Fuel cells convert hydrogen into electricity to run electric motors.

7 Electricity Generation

Hydrogen can be used in fuel cells or gas turbines to generate electricity.

This helps store excess renewable energy.

8 Industrial Applications

Hydrogen is widely used in industries such as:

- petroleum refining

- fertilizer production (ammonia synthesis)

- chemical manufacturing

- steel production

9 Energy Storage

Hydrogen can store excess renewable energy.

Example:

solar energy → hydrogen → electricity later.

This helps balance energy supply and demand.

10 Aerospace Applications

Hydrogen is used as a rocket fuel in space missions.

Example:

liquid hydrogen used in spacecraft propulsion systems.

Advantages of Hydrogen as Energy Carrier

| Advantage | Explanation |
| --- | --- |
| Clean fuel | Produces only water when used |
| High energy content | Contains high energy per unit mass |
| Versatile | Used in transport, electricity, and industry |

Limitations of Hydrogen Use

| Limitation | Explanation |
| --- | --- |
| Storage difficulty | Hydrogen is very light and requires special storage |
| High infrastructure cost | Transport and storage systems are expensive |